/************************************************************
[Annikken Andee - Easter Egg Hunt]
================
Use Bluetooth Signal Strength to Adjust the RGB LED color
Use TTS to Sing a song!
by Robin Ha <robin@annikken.com>

Check out our Resources section for more information and
ideas on what you can do with the Annikken Andee!
http://resources.annikken.com

Check out our Blog for regular updates and fun!
http://annikken.com/blog
************************************************************/

#include <SPI.h>
#include <Andee.h>

// We'll have a displaybox to show you the Bluetooth signal strength
AndeeHelper displaybox;
AndeeHelper displaybox2;
AndeeHelper button;

AndeeHelper speechObject; // You need to create a speech object for the phone to talk
AndeeHelper btnNotify;


int Signals[10] = {0,0,0,0,0,0,0,0,0,0};
int curSignalPos = 0;

int redPin = 13;
int greenPin = 11;
int bluePin = 10;


char strBuffer[30];
char signalStr[4];
int signalStrength;
int state;

// The setup() function is meant to tell Arduino what to do
// only when it starts up.
void setup()
{
    Andee.begin();  // Setup communication between Annikken Andee and Arduino
    Andee.clear();  // Clear the screen of any previous displays
    setInitialData(); // Define object types and their appearance
   
    pinMode(redPin,OUTPUT);
    pinMode(greenPin,OUTPUT);
    pinMode(bluePin,OUTPUT);
   
    Serial.begin(9600);
}

// This is the function meant to define the types and the apperance of
// all the objects on your smartphone
void setInitialData()
{
   
   
    displaybox.setId(0);  // Each object must have a unique ID number
    displaybox.setType(DATA_OUT);  // This defines your object as a display box
    displaybox.setLocation(0,0, FULL); // Sets the location and size of your object
    displaybox.setColor(THEME_YELLOW);
    displaybox.setTitleColor("50000000");
    displaybox.setTitle("EASTER HUNT");
    displaybox.setTitleTextColor("FFFFFFFF");
    displaybox.setData("Where is the egg?");
   
    displaybox2.setId(2);
    displaybox2.setType(DATA_OUT);
    displaybox2.setLocation(1,0,FULL);
    displaybox2.setColor(THEME_ORANGE);
    displaybox2.setTitleColor("50000000");
    displaybox2.setTitle("Egg Detector");
    displaybox2.setUnit("");
   
    button.setId(6); // Don't forget to assign a unique ID number
    button.setType(BUTTON_IN); // Defines object as a button
    button.setLocation(1,0,FULL);
    button.setTitle("Let's celebrate with Easter Bunny Song!");
    button.setColor("FFFFB6C1");
   
    speechObject.setId(12);
    speechObject.setType(TTS); // Defines object as a Text-to-Speech object
   
}
int dcCounter = 3;
bool connection = false;
// Arduino will run instructions here repeatedly until you power it off.
void loop()

{
    if( Andee.isConnected() ) // Run only when connected
    {
        dcCounter = 3;
        if(connection==false){
            connection=true;
            state = 0;
            Andee.clear();
            displaybox.setColor(THEME_YELLOW);
            displaybox.update();
            speechObject.updateData("Welcome to the Easter Egg Hunt!");
            delay(2500);
           
            displaybox2.setData("Initialising.");
           
            speechObject.updateData("Egg Detector initializing");
            displaybox2.update();
           
            delay(700);
           
            displaybox2.setData("Initialising..");
           
            displaybox2.update();
           
            delay(700);
            displaybox2.setData("Initialising...");
            displaybox2.update();
            delay(700);
            displaybox2.setData("Initialising.");
            displaybox2.update();
            delay(700);
           
            speechObject.updateData("Egg Detector Online");
           
            displaybox2.setData("< Online >");
            displaybox2.update();
            delay(1500);
        }
       
        int str = getStrength();
        updateState(str);
        stateToBlinkPattern();
        displaybox.update();
        displaybox2.setData(StateToText());
        displaybox2.update();
        if(state == 6){
            speechObject.updateData("You Have Found the Egg!");
            displaybox2.setData("You have found the egg!");
           
            displaybox2.update();
            spamRandomBlink(3500);
            displaybox2.remove();
            displaybox.setData("Easter Egg Found");
            displaybox.update();
            button.update();
            speechObject.updateData("Lets Sing a Song to celebrate!");
            while(1){
                setRGB(-1,-1,-1,1000);
                if( button.isPressed() )
                {
                    button.ack();
                    button.requireAck(false);
                   
                    button.setTitle("Singing ~ ~");
                    button.update();
                    speechObject.setUtteranceSpeed(0.65);
                    speechObject.setPitch(1.1);
                    speechObject.updateData("This is the Bunny song, Sing along with me!");
                    speechObject.updateData("You put your right paws in");
                    speechObject.updateData("you take your right paws out");
                    speechObject.updateData("You put your right paws in");
                    speechObject.updateData("and you shake them all about");
                    speechObject.updateData("You do the Bunny Pokey");
                    speechObject.updateData("and you hop . all . around");
                    speechObject.updateData("That's what  ");
                   
                    speechObject.setUtteranceSpeed(0.45);
                    speechObject.updateData("  it's all");
                   
                    speechObject.setUtteranceSpeed(0.35);
                    speechObject.updateData("about!");
                    button.setTitle("Singing ~ ~");
                    button.update();
                   
                    spamRandomBlink(12000);
                   
                    button.ack();
                    button.remove();
                    displaybox.setUnit("By Annikken");
                    displaybox.setData("Happy Easter 2014!!");
                    speechObject.setUtteranceSpeed(0.5);
                    speechObject.setPitch(1.0);
                    speechObject.updateData("Happy Easter");
                    for(int y = 0;y<10;y++){
                        displaybox.setColor(THEME_ORANGE);
                        displaybox.update();
                        spamRandomBlink(1500);
                        displaybox.setColor(THEME_GREEN);
                        displaybox.update();
                        spamRandomBlink(1500);
                        displaybox.setColor(THEME_PURPLE);
                        displaybox.update();
                        spamRandomBlink(1500);
                       
                    }
                }
            }
        }
    }
    else
    {
        if(dcCounter>0)
        {
            dcCounter--;
        }
        if(dcCounter==0){
            connection==false;
           
        }
    }
    delay(50);
}

int getStrength(){
    Andee.sendCommand("GET RSSI", signalStr);
    signalStrength = atoi(signalStr); // Convert to int value
    Signals[curSignalPos] = signalStrength;
    curSignalPos++;
    if(curSignalPos>=10){
        curSignalPos=0;
    }
    return calAverageStrength();
   
}
void updateState(int averageStr){
    if(averageStr == 0){
        changeState(0);
    }
    else if (averageStr < -80){
        changeState(1);
    }
    else if (averageStr < -72){
        changeState(2);
    }
    else if (averageStr < -67){
        changeState(3);
    }
    else if (averageStr < -62){
        changeState(4);
    }
    else if (averageStr < -57){
        changeState(5);
    }
    else{
        changeState(6);
    }
}
char * StateToText(){
    if(state == 0){
        return "Calcalating...";
    }
    else if (state == 1){
        return "Still quite far from it";
        displaybox2.setColor("FFFF0000");
    }
    else if (state == 2){
        return "It's getting closer";
        displaybox2.setColor("FF00FF00");
    }
    else if (state == 3){
        return "Within the Vacancy";
       
        displaybox2.setColor("FF0000FF");
    }
    else if (state == 4){
        return "Around the Corner";
        displaybox2.setColor("FFFF00FF");
    }
    else if (state == 5){
        return "Probably Next to you";
       
        displaybox2.setColor("FF00FFFF");
    }
    else if (state == 6){
        displaybox2.setColor(THEME_ORANGE);
        return "You have found the egg!";
    }
}
void changeState (int newState){
    if(newState>state){
        state=newState;
    }
}
void stateToBlinkPattern(){
   
    if(state == 0){
        setRGB(10,10,10,2000);
    }
    else if (state == 1){
        setRGB(255,0,0,2000);
    }
    else if (state == 2){
        setRGB(0,255,0,1500);
    }
    else if (state == 3){
        setRGB(0,0,255,1000);
    }
    else if (state == 4){
        setRGB(255,0,255,1000);
    }
    else if (state == 5){
        setRGB(0,255,255,1000);
    }
    else if (state == 6){
        setRGB(-1,-1,-1,1000);
    }
}

int calAverageStrength(){
    int numberOfValidSignal = 0;
    int TotalStrength;
    for(int x = 0;x<10;x++){
        if(Signals[x]!=0){
            TotalStrength = TotalStrength + Signals[x];
            numberOfValidSignal ++;
        }
    }
    if(numberOfValidSignal==0){
        return 0;
    }
    else{
        int str = TotalStrength/numberOfValidSignal;
        return str;
    }
}
int LR = 0;
int LG = 0;
int LB = 0;
float stepsize_R = 0;
float stepsize_G = 0;
float stepsize_B = 0;
void spamRandomBlink(int duration){
    for (int x  = (duration)/(400);x>=0;x--){
        setRGB(-1,-1,-1,2000);
    }
}
void setRGB(int R, int G, int B,int uDelay){
    if(R==-1){
        R = random(100,255);
    }
    if(G==-1){
        G = random(100,255);
    }
    if(B==-1){
        B = random(100,255);
    }
   
    stepsize_R = (float)(R-LR)/(float)200;
    stepsize_G = (float)(G-LG)/(float)200;
    stepsize_B = (float)(B-LB)/(float)200;
   
    for(int x = 1;x<201;x++){
        analogWrite(redPin, LR+(x*stepsize_R));
        analogWrite(greenPin, LG+(x*stepsize_G));
        analogWrite(bluePin,LB+(x*stepsize_B));
        delayMicroseconds(uDelay);
    }
    LR=R;
    LG=G;
    LB=B;
}